The university of Azad Jammu and Kashmir ,

Muzaffarabad

| Name | Adeela Bashir |
| --- | --- |
| Roll no | 2024-SE-12 |
| Course Title | Computer networks |
| Lab | 10 |
| Submitted to | Dr. Asma javeid |

Department of Software Engineering

Lab# 10

Create Topology

We created a topology with 1 Router 2811, 1 Switch 2960 and 4 PCs. PC0 and PC1 belong to DEV department and PC2 and PC3 belong to HR department.

Create valns on the switch

On the Switch we created two VLANs to separate departments logically:

- VLAN 10 - DEV

- VLAN 20 - HR

Assign PCs to VLANs

We assigned switch ports to VLANs. PC1 and PC2 ports were assigned to VLAN 10 (DEV) and PC3 and PC4 ports were assigned to VLAN 20 (HR).

PC1 and PC2

PC 3 and PC4

Make Switch to Router Link as Trunk

We configured the link between Switch Fa0/1 and Router G0/0 as trunk link using switchport mode trunk command. This trunk link carries traffic of both VLANs to the router.

Configure the router

Configure Router Sub-interfaces (Router-on-a-Stick)

On the Router we created two virtual sub-interfaces on G0/0:

G0/0.10 for VLAN 10 with encapsulation dot1Q 10

G0/0.20 for VLAN 20 with encapsulation dot1Q 20

Each sub-interface was given an IP address to work as default gateway for its VLAN.

Configure VLAN 10 Sub-interface

Configure VLAN 20 sub-interface

Assign IP addresses

Assign IP to PCs

We gave IP addresses to all PCs:

- DEV VLAN 10: 192.168.10.0 /24 network, Gateway 192.168.10.1

- HR VLAN 20: 192.168.20.0 /24 network, Gateway 192.168.20.1

Verify Configuration

We verified the setup using commands:

- show VLAN brief - to check VLANs and assigned ports

- show interfaces trunk - to check trunk link

- show IP interface brief - to check router sub-interfaces are UP

Check the router

Check the VLANs

Check the Trunk:

Test Connectivity with Ping

We tested connectivity using ping.  First we pinged within same VLAN (PC0 to PC1) which was successful. Then we pinged between different VLANs (PC0 of VLAN 10 to PC2 of VLAN 20) through the router.

Test within VLAN 10

Test within VLAN 20

Test inter-VLAN routing

Final Result

The ping between VLAN 10 and VLAN 20 was successful. This proves that Router-on-a-Stick is working and providing Inter-VLAN routing, allowing two different VLANs to communicate with each other.