The university of Azad Jammu and Kashmir ,

Muzaffarabad

| Name | Adeela Bashir |
| --- | --- |
| Roll no | 2024-SE-12 |
| Course Title | Computer networks |
| Lab | 8 |
| Submitted to | Dr. Asma javeid |

Department of Software Engineering

Lab#8

Create Topology:

We take 1 Router 2811 and 2 PCs. We connect PC0 to Router Fa0/0 and PC1 to Router Fa0/1 using Copper Straight-Through cable.

IP Configuration on PCs:

We assign IP to PC0 (192.168.1.2) and PC1 (192.168.2.2). We also set Default Gateway as Router's IP. This gateway is needed to go to other network.

Router CLI  Access:

We click on Router > CLI tab. When it asks "initial configuration dialog [yes/no]" we type NO. Then we go to privileged mode using enable and global config mode using configure terminal.

Interface Configuration:

We select interface Fa0/0 using interface fastEthernet 0/0 and give it IP 192.168.1.1 with ip address command. Then we use no shutdown to ON the interface. Same we do for Fa0/1 with IP 192.168.2.1.

Save Configuration:

We use exit to go back and write memory to save our config so it doesn't delete after restart.

We use show ip interface brief to check if both interfaces are UP UP.

Verification:

We do ping from PC0 to PC1. Ping success means Router is working and routing between two different networks.